Ghost Vehicle Detection in Fog Using Pressure-Wave Sensing Before Visual Detection

C Ganesh Kumar

Assistant Professor

Department of Electronics and Communication Engineering

Amrita School of Engineering

Amrita Vishwa Vidyapeetham , Chennai , India

M Mukesh

Department of Electronics and Communication Engineering

Amrita School of Engineering

Amrita Vishwa Vidyapeetham , Chennai , India

Akshaya M V

Department of Electronics and Communication Engineering

Amrita School of Engineering

Amrita Vishwa Vidyapeetham , Chennai , India

Deva Dharshini P

Department of Electronics and Communication Engineering

Amrita School of Engineering

Amrita Vishwa Vidyapeetham , Chennai , India

Abstract—Vehicle collisions occurring in dense fog remain a significant contributor to road accidents, largely because vision-based detection – whether by a human driver or a camera-based system – degrades sharply once optical contrast falls below a usable threshold. This work presents a Ghost Vehicle Detection prototype that senses an approaching vehicle through the low-frequency pressure waves and acoustic disturbances it produces, combined with direct ultrasonic ranging, so that a warning can be issued before the vehicle is close enough to be seen. The system is built around a Texas Instruments TM4C123GH6PM (ARM Cortex-M4F) microcontroller programmed at the register level in Embedded C, without dependence on vendor abstrac-tion libraries. Three MAX4466 electret microphone modules, arranged around the monitored zone, sense the pressure-wave and acoustic signature of an approaching vehicle’s engine and tyres, while a single HC-SR04 ultrasonic sensor provides a direct, physically grounded distance measurement that is unaffected by fog density. The two sensing modalities are fused asymmetrically: ultrasonic distance drives the proximity classification and alert logic, while the acoustic pressure-wave reading is displayed as a corroborating signal, so that a genuine approach – which registers on both channels – can be distinguished from a spurious reading on either sensor alone. Output is communicated through an SSD1306 OLED display, an LED, and a buzzer reserved for critical proximity. The prototype was validated at reduced scale using a remote-controlled car inside a 35 cm enclosed test box, requiring every threshold to be re-derived for this compressed acoustic and distance scale. This paper documents the complete system architecture, the register-level firmware for every subsys-tem, a series of hardware and firmware faults encountered and resolved during incremental development, and the calibration methodology used, and identifies frequency-domain pressure-wave analysis and true directional localisation as directions for future work.

Index Terms—Ghost Vehicle Detection, Pressure-Wave Sensing, Acoustic Sensing, Ultrasonic Ranging, Sensor Fusion, Fog, Low-Visibility Detection, TM4C123GH6PM, Embedded Systems.

# Introduction

## Fog-Related Vehicle Collisions and Visibility Limitations

Road accidents caused or worsened by dense fog remain one of the most severe categories of vehicular collision, since fog attenuates and scatters visible light in a way that reduces effec-tive contrast roughly in proportion to its density and depth. In many of the worst documented fog-related pile-ups, a chain-reaction collision begins because the lead vehicle in a queue

becomes visible to the vehicle behind it only at a distance too short to permit safe braking. Camera-based and LiDAR-based Advanced Driver Assistance Systems (ADAS), while effective under clear conditions, exhibit well-documented degradation under fog and rain [31], motivating a substantial and recent body of work on fog-robust object detectors [3], [8], [14], [15], [25], [34], fog-aware sensor fusion architectures [1], [4], [24], and simulation-based approaches to studying LiDAR degradation in fog [9], [20]. Even these improved vision-and LiDAR-based pipelines, however, remain fundamentally dependent on an optical or near-optical sensing channel that fog itself directly degrades, motivating sensing approaches that do not depend on light at all.

## Need for Non-Visual Early Warning

A moving vehicle disturbs its surroundings well before it becomes visible, radiating a combination of engine noise, tyre-road interaction noise, and low-frequency pressure-wave dis-turbances that propagate through air largely unaffected by fog, since acoustic and pressure-wave propagation is attenuated by atmospheric particulates and moisture at a rate many orders of magnitude lower than visible light. This insight underlies a substantial body of acoustic and vibration-based vehicle sens-ing research, ranging from long-range acoustic array detection

[10] and stereo-microphone track-before-detect tracking [19], to MEMS microphone-array localisation [27]–[29], fibre-optic distributed acoustic sensing (DAS) of road traffic [11]–[13], [17], [18], [22], and pavement-embedded vibration sensing [26]. These approaches demonstrate that acoustic and pressure-wave signatures carry substantial vehicle-presence information independent of visibility, but the great majority require either an infrastructure-scale fibre-optic installation [11]–[13], [17],

[18], [22], a costly MEMS array [27], [29], or GPU-class deep learning inference [?], [30], placing them out of reach for low-cost, single-vehicle-mounted prototyping.

## Proposed Work

This work presents Ghost Vehicle Detection, an embedded prototype that fuses pressure-wave and acoustic sensing with ultrasonic ranging to detect an approaching vehicle before it is visually resolvable through fog. Three low-cost MAX4466

electret microphone modules, positioned around the monitored zone, sense the pressure disturbance and acoustic signature produced by an approaching vehicle, while a single HC-SR04 ultrasonic sensor – in the same family of proximity sensing exploited for adverse-weather pedestrian detection on heavy vehicles [23] – independently measures physical distance. Rather than relying on either sensor alone, the system fuses the two: ultrasonic distance drives the proximity classification and alert logic, while the acoustic/pressure-wave level is displayed as a corroborating signal, since a genuine approach should register on both channels while a spurious reading on one sensor typically will not. Unlike the DAS, MEMS-array, and deep-learning-based systems surveyed in Section II, the system is implemented entirely with off-the-shelf electret microphones and a single ultrasonic module, running register-level firmware on a low-cost TM4C123GH6PM microcontroller, and is vali-dated at a reduced, miniature scale using a remote-controlled car, with all thresholds re-derived empirically for that scale.

## Objectives

The specific objectives of this work are: (i) to design and construct a multi-sensor embedded hardware prototype com-bining pressure-wave/acoustic sensing with ultrasonic ranging on a single low-cost microcontroller; (ii) to implement every peripheral driver – ADC-based acoustic sampling, I2C-based display control, and Timer-based ultrasonic ranging – at the register level, without vendor abstraction libraries, so that the complete signal path is transparent and auditable; (iii) to design an asymmetric sensor-fusion decision rule in which ultrasonic distance is authoritative for alerting and acoustic level serves only as a corroborating display signal; (iv) to provide a tiered, non-visual warning through an OLED status display, an LED, and a buzzer reserved for critical proximity;

(v) to validate the complete system at a reduced physical scale using a remote-controlled car and to derive an appropriate calibration methodology for that scale; and (vi) to transpar-ently document every hardware and firmware fault encountered during development, on the view that a reproducible account of what went wrong – and why – is as valuable to future embedded-systems work as a description of what ultimately worked.

## Scope and Contributions

The scope of this work is a functional, reproducible proof-of-concept rather than a production-grade or vehicle-certified safety system. Three contributions follow from this scope. First, a fully register-level, dependency-free firmware imple-mentation for a five-peripheral embedded sensor-fusion system on the TM4C123GH6PM, described in sufficient detail (Sec-tion IV) to be independently reproduced. Second, an explicit, asymmetric sensor-fusion design – distance-primary, acoustic-corroborating – justified against the alternative of strict AND-based fusion, and distinguished from the substantially more complex DAS and MEMS-array fusion strategies surveyed in Section II. Third, a transparent account of six non-trivial hardware and firmware faults encountered during incremental

bring-up (Section V-A), together with the diagnostic reasoning used to isolate each one, which is offered as a practical de-bugging reference for similar register-level embedded projects. The remainder of this paper is organised as follows: Section II surveys related work; Section III describes the problem for-mulation and development methodology; Section IV presents the complete system design and firmware; Section V presents results, debugging case studies, and calibration; and Section VI concludes with directions for future work.

# Literature Survey

A survey of thirty-six recent research articles (2023–2026) was conducted spanning vision- and LiDAR-based fog detec-tion, sensor fusion for adverse weather, acoustic array-based vehicle detection, distributed acoustic sensing (DAS), MEMS-based acoustic and vibration sensing, and ultrasonic proximity sensing.

## Vision-Based and Deep-Learning Object Detection in Fog

A large body of recent work adapts YOLO-family object detectors specifically for foggy conditions. Yang et al. pro-posed FogGate-YOLO, using channel-selection mechanisms to improve traffic object detection in foggy environments [3]. Wang et al. developed Fog-YOLO, a real-time detector targeted at foggy autonomous driving scenarios [8]. Wang et al. further presented an improved YOLOv5 variant specifically adapted for foggy-condition vehicle detection [14], while Yuan et al. proposed AWD-YOLO to improve autonomous driving perception reliability across adverse weather more broadly [15]. Jiang et al. introduced YOLO-DH, a robust detector for autonomous vehicles under adverse weather [25], and Cheng et al. proposed an improved YOLOv8 for road object detection in foggy, complex scenes [34]. Fan examined vehicle detection methods specifically under low-light driving scenes [16], and Xie et al. investigated dynamic visibility recognition and driving-risk assessment under combined rain–fog condi-tions using monocular surveillance imagery [5]. Qiu et al. reviewed the broader visual-perception challenges that rain and fog pose for autonomous vehicles [31]. Collectively, this body of work illustrates a consistent pattern: each successive detector variant recovers accuracy lost to fog primarily through architectural modification of the vision pipeline itself (channel attention, adaptive backbones, weather-conditioned augmenta-tion), rather than through the introduction of an independent, fog-immune sensing modality.

## Sensor Fusion and LiDAR-Based Approaches for Fog

Several works fuse vision with LiDAR or radar specifically to counter fog-induced degradation. Zhang et al. proposed an adaptive gated vision-and-LiDAR fusion framework for robust perception in dense fog [1], and Zhang et al. similarly pre-sented FogFusion, a camera-LiDAR fusion approach for robust 3D object detection in foggy autonomous driving [4]. Miao et al. proposed BEVFog to enhance vision-based roadside 3D object detection robustness under foggy conditions [24], while Shi et al. examined multi-target tracking using collaborative

roadside units specifically under foggy conditions [7]. Park et al. used generative adversarial networks to simulate LiDAR behaviour under fog for more robust 3D object detection [9], and Tahir et al. investigated 3D vision-based object detection for autonomous driving in fog using LiDAR [20]. Yuan et al. proposed an ensemble artificial-intelligence framework for real-time automatic vehicle detection and classification [2]. Closer to this work’s low-cost embedded focus, Raza et al. proposed ECE-VDTDA, a computationally efficient collision-avoidance system for driver assistance specifically in foggy weather [6], and separately proposed and assessed an efficient, cost-effective vehicle-detection approach for edge/fog-enabled traffic surveillance in foggy weather [32], [33]. This sub-body of literature is notable for explicitly optimising for computational efficiency at the edge, echoing this work’s own low-cost, single-microcontroller design philosophy, even though the underlying sensing modality remains optical or LiDAR-based rather than acoustic.

## Acoustic Array-Based Vehicle Detection

Acoustic sensing has been explored directly as an alternative or complement to vision for vehicle detection. McKenzie et al. demonstrated long-range vehicle detection and tracking using acoustic arrays [10], and Wang et al. proposed a track-before-detect approach for multiple-vehicle tracking using stereo microphone arrays [19]. Su et al. investigated manipulation of acoustic focusing for multi-target sensing with distributed mi-crophones inside a smart car cabin [21]. Shabbir et al. applied a stacking-based ensemble deep learning approach to acoustic vehicle detection for smart-city traffic management [30], and Ghaffarpasand et al. deployed intelligent acoustic sensors (“Traffic Ear”) alongside vehicle telematics data for traffic-noise assessment [35]. Chakravarthula et al. demonstrated long-range acoustic beamforming for multimodal scene un-derstanding, illustrating the broader potential of sound-based sensing beyond simple detection [?]. A recurring theme across this sub-body of literature is that acoustic sensing performance scales strongly with the number and geometric arrangement of microphones: array- and beamforming-based systems [?], [10], [19], [21] achieve fine-grained localisation and multi-target separation, but at the cost of requiring either a purpose-built array housing or substantial signal-processing compute, both of which this work’s three-microphone, threshold-based design deliberately avoids in favour of low cost and simplicity.

## MEMS Microphone Arrays and Distributed Acoustic Sensing

A growing body of work uses MEMS microphone arrays and fibre-optic distributed acoustic sensing (DAS) for traffic sensing at higher fidelity or larger scale than a single mi-crophone. Zhang et al. developed a high-precision acoustic localisation system for smart cars using a MEMS microphone array [27], and Pecioski et al. developed a low-cost MEMS microphone array for sound localisation in urban environments [29], with Wang et al. reviewing the broader path of MEMS acoustic sensors from research to real-world application [28].

Chang et al. used MEMS acceleration sensors embedded in pavement for vibration-based vehicle detection [26]. On the DAS side, Yi et al. showed that distributed acoustic sens-ing enables real-time traffic monitoring and vehicle motion-state prediction [11], Sun et al. proposed a superlet-enhanced DAS approach for real-time traffic-volume monitoring and vehicle classification [12], and Ma et al. presented a se-mantic perception-topological reasoning framework for robust vehicle-trajectory reconstruction from DAS data [13]. Wu et al. demonstrated real-time, high-precision vehicle-trajectory detection using DAS [17], Yao et al. applied passive DAS signals to urban road-void monitoring [18], and Zhong et al. proposed DAS-enabled smart road infrastructure for intelligent vehicle monitoring [22]. While DAS systems achieve im-pressive traffic-scale monitoring by exploiting existing buried fibre-optic cable as a distributed sensor, this infrastructure dependency – and the accompanying interrogator hardware and signal-processing pipeline – makes DAS unsuitable for a single, standalone, vehicle- or roadside-mountable early-warning unit of the kind targeted in this work.

## Ultrasonic Proximity Sensing

Ultrasonic sensing has also been applied directly to safety-critical proximity detection under adverse weather. Jeong and Kim empirically validated a multidirectional ultrasonic pedestrian-detection system for heavy-duty vehicles specif-ically under adverse weather conditions [23], demonstrat-ing that ultrasonic time-of-flight ranging remains a viable, weather-tolerant sensing modality even where vision-based pedestrian detection would be compromised. This finding directly supports the design choice made in the present work to anchor the proximity-alerting decision on ultrasonic distance rather than on acoustic amplitude alone, since ultrasonic ranging offers a physically direct, weather-tolerant distance measurement that acoustic amplitude, by itself, cannot provide.

## Comparative Positioning

Table I summarises how the surveyed sensing families compare along four practical dimensions relevant to a low-cost, self-contained prototype: approximate hardware cost, dependence on external infrastructure, computational demand, and inherent tolerance to fog.

## Summary and Research Gap

The reviewed literature shows two largely separate trends. On one side, an extensive and rapidly growing body of vision-and LiDAR-based fog-detection research [1], [3], [4], [8],

[9], [14], [15], [20], [24], [25], [31], [34] achieves strong

results but remains fundamentally dependent on an optical or near-optical sensing channel and typically requires GPU-class compute for real-time deep-learning inference. On the other side, acoustic and vibration-based sensing [?], [10], [19], [21], [26]–[30], [35] and distributed acoustic sensing [11]–

[13], [17], [18], [22] demonstrate that non-visual modalities carry substantial vehicle-presence information independent of

TABLE I

Comparative Positioning of Sensing Modalities Surveyed

| Modality | Cost | Infra. De-pendency | Compute | Fog Tol-erance |
| --- | --- | --- | --- | --- |
| Vision / YOLO [3], [8], [14], [15], [25], [34] | Low–Med | None | High (GPU) | Low–Med |
| LiDAR / Fusion [1], [4], [9], [20], [24] | High | None | High | Med–High |
| Acoustic Ar-ray [?], [10], [19], [21] | Medium | None | Medium | High |
| MEMS Array [27]– [29] | Medium | Low | Medium | High |
| DAS [11]– [13], [17], [18], [22] | High | High (fibre) | High | High |
| Ultrasonic [23] | Very Low | None | Very Low | High |
| This Work | Very Low | None | Very Low | High |

fog, but the majority of this work is either infrastructure-scale (fibre-optic DAS cabling, roadside MEMS arrays) or itself deep-learning-heavy, and none of the surveyed acous-tic literature combines a simple, low-cost microphone array with an independent ultrasonic distance measurement in a single self-contained, vehicle-mountable embedded prototype. Ultrasonic sensing has been shown to remain reliable under adverse weather at short range [23], but in isolation offers no corroborating signal to reject spurious echoes. As Ta-ble I makes explicit, this work addresses that specific gap: a low-cost, register-level embedded fusion of simple electret-microphone pressure-wave/acoustic sensing with ultrasonic ranging, requiring no fibre-optic infrastructure, MEMS array, or GPU inference, intended as a pre-visual early-warning prototype rather than a full perception stack.

# Problem Formulation and Methodology

## Problem Formulation

Formally, the objective of this work is to implement a function that maps a set of raw sensor readings – three acoustic ADC channels and one ultrasonic time-of-flight measurement – onto one of four discrete proximity states (MONITORING, CAR DETECTED, APPROACHING, CRITICAL), and to

drive three output actuators (an OLED status display, an LED, and an active buzzer) according to that state, updating contin-uously at a rate fast enough to give an observer meaningful advance warning of an approaching vehicle before it is visually resolvable through fog.

The problem is complicated by three practical constraints identified during development. First, the acoustic sensing mod-

ules used are adjustable-gain electret microphone amplifiers whose output is biased around approximately half the supply voltage rather than centred at zero, meaning a naively-set gain or an incorrect assumption about the expected quiescent ADC reading can make the acoustic channel appear to show a constant high-amplitude signal even in a genuinely quiet environment. Second, the ultrasonic sensor’s echo line outputs a 5V logic signal while the microcontroller’s GPIO pins are rated for 3.3V, requiring either a voltage-divider network or a reduced 3.3V supply to the sensor. Third, validating the system at full vehicle scale, in real fog, was impractical within the available development environment, requiring the entire threshold scheme to be re-derived for a miniature remote-controlled-car test scenario rather than reusing full-scale assumptions.

## Proposed Approach

The proposed approach fuses the two sensing modalities asymmetrically rather than symmetrically: the ultrasonic dis-tance reading is treated as the primary, physically grounded measurement driving the proximity classification and the buzzer/LED alert logic, while the acoustic pressure-wave reading is displayed alongside it as a secondary corroborating indicator rather than being used to gate the alert directly. This design choice was made deliberately after considering the al-ternative of requiring both sensors to agree before triggering an alert: since a test vehicle’s acoustic signature is not guaranteed to be loud at every instant, a strict AND-based fusion risked missing genuine detections, whereas an ultrasonic-primary approach with acoustic corroboration displayed alongside it allows an observer to visually cross-check the two readings without the system silently suppressing a valid distance-based alert. This design deliberately trades the higher localisation fidelity of the array- and DAS-based systems surveyed in Section II [11]–[13], [27] for a self-contained, single-board implementation requiring no external infrastructure or multi-channel beamforming pipeline.

## Development Methodology

The project was developed using an incremental, bottom-up bring-up methodology, in which each hardware subsystem and its corresponding firmware driver was implemented and independently verified before being integrated with the next. This sequence proceeded as follows: (1) LED blink test to confirm the toolchain, debugger, and board were functioning;

(2) single-channel ADC read to confirm analog sensing; (3) I2C and SSD1306 OLED driver bring-up, including a com-plete 5x7 pixel font table; (4) extension of the ADC driver to three simultaneously sampled acoustic channels; (5) buzzer and LED GPIO driver bring-up; (6) HC-SR04 ultrasonic driver bring-up using a Timer0-based microsecond-resolution time-of-flight measurement; and (7) integration of all subsystems into a single fusion firmware with threshold-based alerting. This incremental approach proved essential in practice, since it allowed each of the several hardware and firmware faults en-countered (Section V-A) to be isolated to a specific, recently-

1. LED Blink Test

2. Single-Channel ADC Read

3. I2C + OLED Bring-Up

4. Three-Channel ADC Extension

5. Buzzer + LED Bring-Up

6. Ultrasonic Driver Bring-Up

7. Full Sensor-Fusion Integration

Fig. 1. Incremental bring-up methodology: each stage was independently verified before the next was added.

added subsystem rather than requiring debugging of the entire integrated system at once. Fig. 1 summarises this sequence.

## Tools and Technologies

Hardware: a Texas Instruments EK-TM4C123GXL LaunchPad (TM4C123GH6PM, ARM Cortex-M4F); three MAX4466-type adjustable-gain electret microphone modules; one HC-SR04 ultrasonic module; a 0.96-inch SSD1306 I2C OLED (128x64, address 0x3C); an active buzzer; and a discrete LED with current-limiting resistor. Software: Keil MDK v5 (uVision) as the IDE and ARM compiler toolchain; Embedded C written at the register level directly against the TM4C123GH6PM peripheral memory map, without TivaWare; and the onboard Stellaris In-Circuit Debug Interface (ICDI) for flashing and debugging over USB. Table II lists the on-chip peripherals used and their role.

# System Design and Implementation

## System Architecture

The system follows a linear sense-process-actuate architec-ture. Three acoustic channels and one ultrasonic channel are sampled every control-loop iteration; the ultrasonic reading is classified into one of four proximity bands; the maxi-mum of the three acoustic readings is computed as a coarse corroborating level indicator; and the resulting state drives the OLED, LED, and buzzer outputs. The loop repeats at

TABLE II

Peripheral Modules Used

| Peripheral | Purpose |
| --- | --- |
| ADC0 (SS1) | Simultaneous 12-bit sampling of three acoustic channels |
| I2C0 | Communication with the SSD1306 OLED |
| GPIO Port B | OLED (I2C), buzzer, LED, ultrasonic TRIG/ECHO |
| GPIO Port E | Analog inputs for the three microphone modules |
| Timer0 | Free-running microsecond timer for ul-trasonic ranging |

3x MAX4466 Pressure-Wave/Acoustic Mics

+ HC-SR04 Ultrasonic Sensor

ADC0 (3-Ch Acoustic) + Timer0 (Ultrasonic Time-of-Flight)

Sensor Fusion:

Distance Primary, Acoustic Corroborates

4-Band Proximity Classification (Monitoring / Detected / Approaching / Critical)

OLED Status + LED + Buzzer (Buzzer Reserved for Critical)

Fig. 2. Overall architecture of the Ghost Vehicle Detection system.

approximately 150 ms, providing a responsive OLED update rate without saturating the I2C bus with excessive redraw traffic, as summarised in Fig. 2.

At the hardware level, three MAX4466 acoustic sensors feed ADC0’s second sample sequencer (SS1) on GPIO Port E; one HC-SR04 ultrasonic sensor is interfaced via GPIO Port B using a software trigger pulse and a Timer0-timed echo measurement; an SSD1306 OLED is driven over I2C0 on Port B; and a buzzer and LED are driven as simple digital GPIO outputs, also on Port B.

## Hardware Components

  - TM4C123GH6PM LaunchPad: the EK-TM4C123GXL,

built around the Cortex-M4F-based TM4C123GH6PM, serves as the central processing platform, selected for its low cost, onboard USB debug interface, and rich on-chip peripherals used directly at the register level rather than through a vendor abstraction layer.

  - MAX4466 Acoustic Sensors: three adjustable-gain electret microphone modules provide acoustic sensing at three physical positions (Front, Back-Right, Back-Left, spaced approximately

TABLE III PIN MAPPING

| Component | Signal | Pin |
| --- | --- | --- |
| Mic 1 (Front) | Analog Out | PE3 (AIN0) |
| Mic 2 (Back-Right) | Analog Out | PE2 (AIN1) |
| Mic 3 (Back-Left) | Analog Out | PE1 (AIN2) |
| OLED | SDA / SCL | PB3 / PB2 |
| Buzzer | Signal | PB5 |
| LED | Anode (via R) | PB4 |
| Ultrasonic | TRIG / ECHO | PB6 / PB7 |

120°), in the same general spirit as low-cost microphone-array localisation work [27], [29] but without a dedicated MEMS array or beamforming pipeline [?]. Each module outputs an analog voltage biased around half the supply voltage, riding an AC audio signal proportional to sound pressure level. An onboard gain-trim potentiometer makes sensitivity adjustable, which proved directly relevant to a calibration issue discussed in Section V-C.

  - HC-SR04 Ultrasonic Sensor: provides the primary distance measurement, following the same weather-tolerant proximity-sensing principle validated for heavy-vehicle pedes-trian detection [23]. It emits a 40 kHz burst upon a trigger pulse of at least 10 µs and returns an echo pulse whose duration is proportional to round-trip time of flight.

  - SSD1306 OLED Display: a 128x64 monochrome display driven over I2C at address 0x3C presents live distance, acous-tic level, and textual status, chosen over a character LCD for its lower pin count and pixel-addressable text.

  - Buzzer and LED: an active buzzer produces sound con-tinuously while driven high; a discrete LED through a current-limiting resistor provides a lower-urgency visual indicator preceding the buzzer.

## Circuit Design and Connections

Table III lists the complete pin allocation. Since the HC-SR04’s echo output nominally swings to 5V while the micro-controller’s GPIO inputs are rated for 3.3V, direct connection risks damaging the input pin. Two accommodations were considered: a resistive voltage-divider network (1 kΩ in series, 2 kΩ to ground) when the sensor is powered at 5V; or powering the sensor directly from the 3.3V rail, causing its echo output to swing only to approximately 3.3V and removing the need for the divider network. The latter was adopted after a usable 5V pin could not be readily identified on the LaunchPad header, and was found to produce usable readings for the short ranges relevant to this project, at the cost of operating the sensor outside its officially specified supply range.

## Firmware Architecture

The firmware was structured into independent, single-responsibility modules, each with a header declaring its public interface and a source file containing its implementation. The modules implemented were: delay (millisecond software delay), adc (three-channel acoustic sampling), i2c (I2C0

master driver), oled (SSD1306 display driver, including a complete 5x7 font table), buzzer, led, and ultrasonic (HC-SR04 trigger/echo timing driver), with main.c imple-menting the top-level control loop and fusion/alerting logic.

## Acoustic Sensing Module (ADC0)

The three acoustic channels are sampled using ADC0’s Sample Sequencer 1 (SS1), configured to sample AIN0 (PE3, Front), AIN1 (PE2, Back-Right), and AIN2 (PE1, Back-Left) in a single triggered sequence, with the END flag set on the third and final sample. This configuration allows all three channels to be read with a single software trigger and a single polling wait, rather than requiring three separate single-channel conversions.

void ADC0_Init3Ch(void) { SYSCTL->RCGCADC |= (1<<0);

SYSCTL->RCGCGPIO |= (1<<4); GPIOE->AFSEL |= 0x0E;

GPIOE->DEN &= ˜0x0E; GPIOE->AMSEL |= 0x0E; ADC0->ACTSS &= ˜(1<<1); ADC0->EMUX &= ˜0x00F0;

ADC0->SSMUX1 = 0x00000210; ADC0->SSCTL1 = (1<<10);

ADC0->ACTSS |= (1<<1);

}

Listing 1. ADC0 three-channel initialisation

An earlier version of this module used the channel-order value 0x0123 instead of 0x0210, which inadvertently sam-pled AIN3 (an unconnected pin, PE0) as the first channel rather than the intended Front microphone on AIN0. Because a floating analog input on this microcontroller tends to set-tle near the upper end of the conversion range rather than fluctuating randomly, this bug manifested as a consistently high, direction-detection-dominating reading that was initially mistaken for an acoustic calibration problem rather than a channel-mapping error, as discussed further in Section V-A.

## I2C and OLED Display Module

The OLED display is driven over I2C0, configured for approximately 100 kHz operation, using PB2 as the clock line (SCL) and PB3 as the open-drain data line (SDA). The display driver implements the standard SSD1306 initialisation command sequence (charge pump enable, segment remap, COM scan direction, contrast setting, and display-on), fol-lowed by page-addressed writes for clearing the screen and rendering text using a complete 5x7 pixel font table covering the printable ASCII range from space (32) through underscore (95).

static void OLED_Command(uint8_t cmd) { I2C0->MSA = (OLED_ADDR << 1);

I2C0->MDR = 0x00; // control byte: cmd

I2C0->MCS = 0x03; // START+RUN

while(I2C0->MCS & 0x01); I2C0->MDR = cmd;

I2C0->MCS = 0x05; // RUN+STOP

while(I2C0->MCS & 0x01);

}

Listing 2. OLED command/data transaction primitive

A build-time issue encountered during OLED driver devel-opment is worth noting for completeness: the OLED source file directly accessed I2C0 peripheral registers without in-cluding the microcontroller’s device header, which compiled successfully in files that already included that header tran-sitively but failed with undefined-symbol errors in oled.c specifically, since the register struct definitions were never pulled in. This was resolved by explicitly including the device header at the top of oled.c.

## Ultrasonic Ranging Module

The ultrasonic driver uses Timer0, configured as a free-running 32-bit up-counter, to measure the duration of the echo pulse with microsecond resolution. A trigger pulse of at least 10 µs is generated on PB6 using a Timer0-derived microsecond delay function (replacing an earlier, unreliable instruction-count busy-wait loop that could not guarantee a consistent pulse width across compiler optimisation settings), after which the driver polls PB7 for a rising edge, records the timer value, polls for the corresponding falling edge, and computes the distance from the elapsed tick count:

t (µs)

TABLE IV

Distance Threshold Mapping for the 35 cm Test Box

| Distance | Status | LED / Buzzer |
| --- | --- | --- |
| >30 cm | Monitoring | Off / Off |
| 20–30 cm | Car Detected | On / Off |
| 10–20 cm | Approaching | On / Off |
| <10 cm | Critical | On / On |

void UpdateAlerts(uint16_t distCM) {

if (distCM > 30) { Buzzer_Off(); LED_Off(); } else if (distCM > 10) { Buzzer_Off(); LED_On(); } else { Buzzer_On(); LED_On(); }

}

Listing 4. Fusion and alerting logic (main.c excerpt)

The acoustic level (the maximum of the three ADC channel readings) is computed and displayed on the OLED alongside distance but does not itself gate the alert outputs, serving instead as a visual corroboration channel for the observer. This design deliberately trades the higher fidelity of DAS-or MEMS-array-based localisation [11]–[13], [27] for a self-contained, single-board implementation requiring no external infrastructure.

d (cm) =

58

(1)

## I. Miniature-Scale Adaptation

Listing 3. Ultrasonic distance measurement

A timeout mechanism is included on both the rising-edge and falling-edge polling loops, returning a sentinel value of 0xFFFF (interpreted downstream as “NO SIGNAL”) if no valid echo is received within a bounded number of polling iterations. This was essential during development, since an unconnected or disconnected ultrasonic sensor leaves the echo pin electrically floating, and a floating digital input can register spurious, noise-driven transitions that would otherwise be misinterpreted as a very short, and therefore very close, echo pulse – a fault documented in detail in Section V-A.

## Sensor Fusion and Alerting Logic

The fusion logic adopted in this project is intentionally simple and interpretable: the ultrasonic distance reading is classified into one of four bands (Table IV), and the LED and buzzer are driven directly from this classification, with the buzzer reserved exclusively for the CRITICAL band:

⎪⎨⎧Off / Off  d > 30 cm (Monitoring)

Because full-scale, real-fog validation was outside the prac-tical scope of this work, the system was instead validated using a remote-controlled car operating inside an enclosed cardboard test box approximately 35 cm in length, with the sensor array mounted at one end and an opening at the other through which the RC car enters. This required two significant adaptations to the design as originally conceived for full-scale vehicles. First, an RC car’s electric motor produces substantially less acoustic and pressure-wave energy, and at a higher character-istic frequency, than a full-scale internal combustion engine, meaning the amplitude thresholds appropriate for a full-scale vehicle at a range of metres are entirely inapplicable at RC-car scale and centimetre-range distances. Second, the distance thresholds themselves (Table IV) had to be compressed from a metre-scale range down to the 35 cm box, since the ultrasonic sensor’s own minimum reliable range (approximately 2 cm) becomes a non-trivial fraction of the total usable range at this scale. Both sets of thresholds were established empirically rather than by reusing full-scale assumptions, following the calibration methodology described in Section V-C.

uint16_t Ultrasonic_GetDistanceCM(void) { GPIOB->DATA &= ˜TRIG_PIN;

Delay_us(2);

GPIOB->DATA |= TRIG_PIN;

Delay_us(10);

GPIOB->DATA &= ˜TRIG_PIN;

/* wait ECHO rising edge (timeout->0xFFFF) */

/* wait ECHO falling edge (timeout->0xFFFF) */

/* return pulse_us / 58 */

}

Results and Discussion

## Development and Debugging Process

The incremental bring-up methodology described in Section III-C surfaced a series of discrete faults, each isolated and re-solved before proceeding to the next integration step. These are documented here both as a record of the development process and because several of them illustrate debugging principles of

Alert(d) =

On / Off	10 < d ≤ 30 cm

⎪⎩On / On	d ≤ 10 cm (Critical)

(2)

general relevance to embedded systems development.

  - Compiler Language-Mode Errors: on first attempting to build the fully integrated firmware, the Keil ARM compiler

reported errors such as “type name is not allowed” and “dec-laration may not appear after executable statement in block” at every instance of a for-loop with an inline variable declaration. This was traced to Keil’s default strict C90 language mode, which does not permit variable declarations inside a for-statement or after other statements within a block – both of which are permitted under C99, the standard the firmware was written against. The fix was to explicitly enable C99 mode in the project’s compiler settings.

  - Undefined Register Symbols in oled.c: the OLED driver file failed to compile with undefined-symbol errors on I2C0 register accesses, despite the identical register accesses com-piling successfully elsewhere in the project, because oled.c included only its own header and the I2C driver’s header, neither of which pulls in the peripheral register definitions. Adding an explicit device-header include resolved the issue.

  - Floating ADC Channel: after extending the ADC driver from one to three channels, the system consistently reported very high acoustic levels and a fixed “Front” direction regard-less of actual ambient sound, even in a quiet room. Diagnosis revealed that the ADC sample-sequencer channel-order regis-ter (SSMUX1) had been configured with the value 0x0123, which samples AIN3 (an entirely unconnected pin) as the first channel rather than AIN0 (the intended Front microphone). Correcting the register value to 0x0210 resolved the channel-mapping error, though a separate, still-elevated baseline offset on all three genuine channels was identified afterward and is discussed in Section V-C.

  - Linker Error from Duplicate main() Definitions: a “sym-bol main multiply defined” linker error occurred after a diagnostic version of main.c (saved as main_full.c for backup purposes) was left included in the Keil project’s Source Group alongside the active main.c. The fix was to remove the unused file from the Source Group.

  - Stellaris ICDI Driver Detection Failure: on first attempt-ing to flash the compiled firmware, Keil reported “Could not find an Cortex-M device in the JTAG chain,” and the debugger’s Attached Devices list was empty. Windows Device Manager showed the board’s In-Circuit Debug Interface entries under “Other devices” with driver-missing warning indicators, alongside a working virtual COM port entry, indicating that Windows had detected the USB device electrically but had no driver bound to its debug interface. This was resolved by manually installing the official Stellaris ICDI driver package via Device Manager’s “Browse my computer for drivers” option, rather than relying on Windows’ automatic online driver search, which returned no results for this legacy driver.

  - Floating Ultrasonic Echo Line: after adding the ultra-sonic ranging subsystem to the fusion firmware, the buzzer was observed to sound continuously regardless of actual proximity. A multimeter continuity check ruled out an electrical short between the buzzer’s positive terminal and the power rail, confirming the buzzer circuit itself was wired correctly. The actual cause was that the ultrasonic sensor had not yet been physically wired into the circuit at that stage of testing, leaving the echo input pin (PB7) electrically floating. A

floating digital input does not settle to a stable logic level and can register spurious, noise-driven high-to-low transitions; the ranging driver interpreted these as an extremely short (and therefore extremely close) echo pulse on nearly every polling cycle, driving the fusion logic into the CRITICAL band continuously. Wiring in the physical ultrasonic sensor resolved the false readings; a subsequent firmware improvement also rejects implausibly short pulse widths as a defensive measure against a disconnected or floating sensor in the future. Table V summarises all six faults.

## Functional Testing Results

Following resolution of the faults above, each subsystem was independently re-verified before final integration, consis-tent with the incremental methodology adopted. The OLED correctly displayed static startup text and, subsequently, live-updating numeric values. Each of the three acoustic channels was confirmed to respond independently to a tap or clap near its corresponding microphone, with no cross-channel interfer-ence observed once the channel-mapping fix was applied. The ultrasonic sensor, once physically wired and powered from the 3.3V rail with a direct (non-divided) echo connection, produced stable, non-“NO SIGNAL” distance readings during bench testing.

## Calibration Results

Distance thresholds for the 35 cm test box (Table IV) were established through direct observation of the ultrasonic reading with the RC car positioned at the box opening, at the box midpoint, and directly at the sensor. Acoustic channel calibration proved more involved: raw quiet-room readings on all three channels were observed in the range of approximately 3100–3300 (on a 0–4095 twelve-bit scale), noticeably higher than the approximately 2048 mid-supply bias point expected from these microphone modules at rest. Ambient room fans were tested and ruled out as the primary cause, since disabling them produced no meaningful change in the readings. At the time of writing, the most probable remaining causes – an excessively high gain-trim potentiometer setting on each mi-crophone module, or the modules being inadvertently powered from a 5V rather than 3.3V supply – had been identified but not yet conclusively isolated, and this is recorded as an open calibration item rather than a resolved result.

## Limitations

Four limitations are worth stating explicitly. First, the acoustic baseline offset described in Section V-C remained unresolved at the time of writing, meaning the acoustic “SND” reading, while displayed and directionally indicative, was not yet reliable as a calibrated corroboration signal for the alerting logic. Second, running the HC-SR04 sensor at 3.3V rather than its specified 5V supply, while found to work for the short ranges used in this project, is outside the manufac-turer’s specified operating conditions and may not generalise reliably to a full-scale deployment requiring the sensor’s full rated range. Third, the three-microphone amplitude-maximum

TABLE V

Summary of Hardware and Firmware Faults Identified and Resolved

| # | Fault | Symptom | Resolution |
| --- | --- | --- | --- |
| 1 | C90/C99 compiler mis-match | “type name is not allowed” on every for-loop with inline declaration | Enabled C99 mode in compiler settings |
| 2 | Missing device-header in-clude | Undefined-symbol errors on I2C0 registers in oled.c only | Added explicit device-header include to oled.c |
| 3 | Floating ADC channel | Consistently high acoustic level, fixed direction reading | Corrected SSMUX1 from 0x0123 to 0x0210 |
| 4 | Duplicate main() defini-tions | “symbol main multiply defined” linker error | Removed backup file from Keil Source Group |
| 5 | Missing Stellaris ICDI driver | “Could not find a Cortex-M device in the JTAG chain” | Manually installed driver via Device Man-ager |
| 6 | Floating ultrasonic echo line | Buzzer sounded continuously regardless of prox-imity | Physically wired in the sensor; added minimum-pulse-width check |

TABLE VI

Raw Acoustic ADC Readings Before and After the

Channel-Mapping Fix

| Condition | Front | Back-R | Back-L |
| --- | --- | --- | --- |
| Before fix (floating pin sampled first) | >2800 (fixed) | – | – |
| After fix, quiet, fans on | 3345 | 3173 | 3125 |
| After fix, quiet, fans off | ∼same | ∼same | ∼same |

approach provides only a coarse indication of the dominant sound direction and is expected to be ambiguous for a vehicle approaching from a bearing roughly equidistant between two microphones – a limitation that dedicated MEMS-array [27],

[29] or DAS-based [13], [17] systems address at substantially higher cost and infrastructure complexity. Fourth, the HC-SR04’s approximately 15° detection cone, combined with the confined geometry of the enclosed test box, introduces a risk of spurious echoes from the box walls rather than the RC car itself, a risk that is reduced but not eliminated by the acoustic corroboration display.

# Conclusion and Future Work

This paper presented Ghost Vehicle Detection, a low-cost embedded prototype that fuses pressure-wave/acoustic sensing with ultrasonic ranging to warn of an approaching vehicle in fog before it becomes visually resolvable. A working proto-type was constructed around a TM4C123GH6PM LaunchPad, three MAX4466 acoustic sensor modules, and an HC-SR04 ultrasonic sensor, with all peripheral drivers implemented at the register level in Embedded C. The system successfully demonstrates tiered, distance-based alerting through an OLED status display, an LED, and a critical-proximity buzzer, and was validated at a reduced scale using a remote-controlled car inside an enclosed test box, with detection thresholds specifically re-derived for this scale rather than reused from full-scale assumptions. As Table I makes explicit, this design occupies a distinct point in the design space relative to the

vision, LiDAR, acoustic-array, MEMS-array, and DAS-based systems surveyed in Section II: substantially lower cost and computational demand, at the expense of coarser directionality and shorter effective range.

The development process surfaced and resolved a series of instructive hardware and firmware faults – a compiler language-mode mismatch, an undefined-register compilation error, an incorrect ADC channel-mapping configuration, a duplicate-symbol linker error, a missing debugger driver, and a floating-input false-alarm condition on the ultrasonic echo line – each diagnosed through the same incremental, subsystem-by-subsystem verification methodology adopted from the outset. The project also surfaced a genuine open limitation: an unre-solved elevated baseline offset on all three acoustic channels, whose most likely causes were identified but not conclusively isolated at the time of writing, reported transparently as an item for continued work.

Future work includes: (i) resolving the outstanding acoustic baseline calibration issue so the acoustic channel can be used as an active corroborating gate on the alert logic rather than a display-only indicator; (ii) replacing the simple amplitude-threshold acoustic detection with frequency-domain analysis, informed by the acoustic classification approaches surveyed in Section II [30], [35], to distinguish genuine vehicle signatures from broadband ambient noise; (iii) extending the ultrasonic ranging subsystem to operate at its manufacturer-specified 5V supply with a proper voltage-divider or level-shifting circuit;

(iv) investigating true directional localisation, drawing on the MEMS-array and beamforming techniques surveyed in Section II [?], [27], [29], rather than the coarse maximum-amplitude direction indication used in this prototype; (v) exploring low-cost distributed sensing inspired by DAS-based traffic monitoring [11], [22] at a fraction of the infrastructure cost;

(vi) validating the system at full vehicle scale in a real fog environment, alongside the vision- and LiDAR-based baselines surveyed in Sections II-A and II-B [3], [4], [8], [15]; and (vii) exploring multi-sensor voting or probabilistic sensor fusion as a more principled alternative to the current rule-based design.

References

- Zhang, Fengyuan, Zixuan Guo, Jianbo Ding, Jingyun Yang, and Wenhe Liu. “Adaptive sensor fusion for robust perception in dense fog: A gated vision and LiDAR integration framework.” Sensors 26, no. 12 (2026): 3728.

- Yuan, Quan, Hyunwook Kwon, Mukhlis Raza, Soojeong Lee, Ali Alqahtani, Jamil Hussain, Saied Salem, Yeong Hyeon Gu, and Mugahed

A. Al-Antari. “Ensemble Artificial Intelligence Framework for Real-Time Automatic Vehicle Detection and Classifications.” IEEE Access (2026).

- Yang, Yuhe, Suilian You, Jinpeng Yu, and Bo Lu. “FogGate-YOLO: Traffic Object Detection in Foggy Environments Using Channel Selec-tion Mechanisms.” Sensors 26, no. 6 (2026): 1811.

- Zhang, Bingli, Yixin Wang, Chengbiao Zhang, Junzhao Jiang, Xiang Luo, Xinyu Wang, Yangyang Zhang, et al. “FogFusion: Robust 3D object detection based on camera-LiDAR fusion for autonomous driving in foggy weather conditions.” Proceedings of the Institution of Mechanical Engineers, Part D: Journal of Automobile Engineering 240, no. 4 (2026): 2312–2322.

- Xie, Zilong, Chi Zhang, Dibin Wei, Xiaomin Yan, and Yijing Zhao. “Dynamic Visibility Recognition and Driving Risk Assessment Under Rain–Fog Conditions Using Monocular Surveillance Imagery.” Sustain-ability 18, no. 2 (2026): 625.

- Raza, Naeem, Muhammad Asif Habib, Abdullah M. Albarrak, Mudassar Ahmad, Alaa Eldeen Sayed Ahmed, Muhammad Yasir, Habib Ur Rahman, and Muhammad Ahsan Latif. “ECE-VDTDA: A robust and computationally efficient collision avoidance system for driver assistance in foggy weather.” PLOS ONE 21, no. 2 (2026): e0342186.

- Shi, Tao, Xuan Wang, Wei Jiang, Xiansheng Huang, Ming Cen, Shuai Cao, and Hao Zhou. “Multi-Target Tracking with Collaborative Roadside Units Under Foggy Conditions.” Sensors 26, no. 3 (2026): 998.

- Wang, Quanxiang, Zhaofa Zhou, and Zhili Zhang. “Fog-YOLO: A Real-Time Object Detector for Foggy Autonomous Driving Scenarios.” Symmetry 18, no. 2 (2026): 322.

- Park, Yeonsoo, Yoonho Cho, Jisub Kwak, Yejin Park, and Jaewan Lee. “Simulation of LiDAR under fog with generative adversarial networks for robust 3D object detection.” International Journal of Automotive Technology 27, no. 2 (2026): 883–895.

- McKenzie, Kevin, Thomas Watson, and Eddie Jacobs. “Long range vehicle detection and tracking using acoustic arrays.” In Autonomous Systems: Sensors, Processing, and Security for Ground, Air, Sea, and Space Vehicles and Infrastructure 2026, vol. 14044, p. 140440C. SPIE, 2026.

- Yi, Ke, Jingming Zhang, Yuanhang Zhuo, Jiayao Sun, Deyu Xu, Shanfan Lu, Xingjie Lai, Liyang Shao, and Ning Zhu. “Distributed Acoustic Sensing Enables Real-Time Traffic Monitoring and Vehicle Motion-State Prediction.” iOptics (2026): 100034.

- Sun, Qingshuo, Yaozhang Sai, Yongqiang Song, Junyao Wang, Tong Feng, Yaning Huang, Haiyang Wang, Xiaozuo Liu, and Lili Wang. “Superlet-enhanced distributed acoustic sensing for real-time traffic vol-ume monitoring and vehicle classification.” Optics & Laser Technology 201 (2026): 115310.

- Ma, Xianyong, Yufan Wang, Guowei Li, Zejiao Dong, Qilin Huang, Gershome G. Abaho, and Wei Cao. “A semantic perception-topological reasoning framework for robust vehicle trajectory reconstruction in distributed acoustic sensing.” Expert Systems with Applications (2026): 131886.

- Wang, S. F., Y. X. Lei, F. X. Sun, and J. Y. Zhang. “Adaptive vehicle detection in foggy conditions based on improved YOLOv5.” Advances in Transportation Studies 69 (2026): 121.

- Yuan, Ya, Wanli Dong, Sicong Yang, and Tianya Wu. “AWD-YOLO enhancing autonomous driving perception reliability in adverse weather.” Scientific Reports 16, no. 1 (2026): 338.

- Fan, Jiacheng. “Autonomous Driving Vehicle Detection Methods under Different Low-Light Scenes.” In Proceedings of the International Work-shop on Advances in Deep Learning for Image Analysis and Computer Vision (IWADIC 2025), p. 154. Springer Nature, 2026.

- Wu, Huijuan, Zhichao Wang, Yuanyuan Song, Anjie Sun, Chenhui Huang, Shibo Han, Yu Wu, and Yunjiang Rao. “Real-Time High-Precision Detection of Vehicle Trajectories Using DAS.” Photonic Sensors 16, no. 1 (2026): 9560008.

- Yao, Shuai, Qiwei Liu, Yixuan Dong, Xiaobei Yan, Zeyuan Fan, Jingze Du, Zheng Yang, Wenquan Ouyang, Qisong Wu, and Dong Wu. “Urban

road void monitoring using passive distributed acoustic sensing signals.” Measurement (2026): 122786.

- Wang, Jiazheng, Nida Ishtiaq, Amirali Khodadadian Gostar, Reza Ho-seinnezhad, and Mohammad Fard. “Track-Before-Detect Based Multiple Vehicle Tracking with Stereo Microphone Arrays.” SSRN 7150719 (2026).

- Tahir, Alishba, Rafia Mumtaz, and Muhammad Saqib Irshad. “3D vision object detection for autonomous driving in fog using LiDaR.” Simulation Modelling Practice and Theory 140 (2025): 103089.

- Su, Yuqi, Fusang Zhang, Beihong Jin, and Daqing Zhang. “Manipulation of acoustic focusing for multi-target sensing with distributed micro-phones in smart car cabin.” Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies 9, no. 2 (2025): 1–28.

- Zhong, Ruikang, Chia-Yen Chiang, and Mona Jaber. “Intelligent vehicle monitoring: Distributed-acoustic-sensor-enabled smart road infrastruc-ture.” IEEE Internet of Things Journal 12, no. 11 (2025): 15211–15223.

- Jeong, Hyeon-Suk, and Jong-Hoon Kim. “Empirical validation of a multidirectional ultrasonic pedestrian detection system for heavy-duty vehicles under adverse weather conditions.” Sensors 25, no. 17 (2025): 5287.

- Miao, Gaoyuan, Wentong Li, Rong Quan, and Jie Qin. “BEVFog: Enhancing Vision-Based Roadside 3D Object Detection Robustness under Foggy Conditions.” In ECAI 2025: 28th European Conference on Artificial Intelligence, 3591–3598. SAGE Publications, 2025.

- Jiang, Liye, Guanqun Ma, Weixuan Guo, and Yunhao Sun. “YOLO-DH: Robust Object Detection for Autonomous Vehicles in Adverse Weather.” Electronics 14, no. 22 (2025): 4476.

- Chang, Congyi, Linghui Kong, Libin Han, Junmin Li, Shuo Pan, and Ya Wei. “Vibration analysis and vehicle detection by MEMS acceleration sensors embedded in PCC pavement.” Sensors 25, no. 9 (2025): 2898.

- Zhang, Qiyan, Wensheng Sun, Jiayi Wang, Fengrui Yang, Rui Long, and Hao Wu. “High-Precision Acoustic Localization System for Smart Cars Using IM68A130 MEMS Microphone Array.” In 2025 Interna-tional Conference on Aerospace Information Perception and Intelligent Processing (AIPIP), 214–217. IEEE, 2025.

- Wang, Qingyi, Yang Zhang, Sizhe Cheng, Xianyang Wang, Shengjun Wu, and Xufeng Liu. “MEMS acoustic sensors: Charting the path from research to real-world applications.” Micromachines 16, no. 1 (2024):

43.

- Pecioski, Damjan, Simona Domazetovska Markovska, Viktor Gavriloski, Maja Anachkova, and Anastasija Ignjatovska Angjusheva. “Develop-ment of low-cost MEMS microphone array for sound localization in urban environments.” In INTER-NOISE and NOISE-CON Congress and Conference Proceedings, vol. 270, no. 7, 4641–4651. Institute of Noise Control Engineering, 2024.

- Shabbir, Ahsan, Ammara Nawaz Cheema, Inam Ullah, Ibrahim M. Almanjahie, and Fatimah Alshahrani. “Smart city traffic management: Acoustic-based vehicle detection using stacking-based ensemble deep learning approach.” IEEE Access 12 (2024): 35947–35956.

- Qiu, Yongsheng, Yuanyao Lu, Yuantao Wang, and Chaochao Yang. “Vi-sual perception challenges in adverse weather for autonomous vehicles: A review of rain and fog impacts.” In 2024 IEEE 7th Information Technology, Networking, Electronic and Automation Control Conference (ITNEC), vol. 7, 1342–1348. IEEE, 2024.

- Raza, Naeem, Muhammad Asif Habib, Mudassar Ahmad, Qaisar Abbas, Mutlaq B. Aldajani, and Muhammad Ahsan Latif. “Efficient and cost-effective vehicle detection in foggy weather for edge/fog-enabled traffic surveillance and collision avoidance systems.” Computers, Materials and Continua 81, no. 1 (2024): 911–931.

- Raza, Naeem, Mudassar Ahmad, and Muhammad Asif Habib. “Assess-ment of efficient and cost-effective vehicle detection in foggy weather.” In 2024 18th International Conference on Open Source Systems and Technologies (ICOSST), 1–6. IEEE, 2024.

- Cheng, Long, Dan Zhang, and Yan Zheng. “Road object detection in foggy complex scenes based on improved YOLOv8.” IEEE Access 12 (2024): 107420–107430.

- Ghaffarpasand, Omid, Anwar Almojarkesh, Sophie Morris, Elizabeth Stephens, Alaa Chalabi, Usamah Almojarkesh, Zenah Almojarkesh, and Francis D. Pope. “Traffic noise assessment using intelligent acoustic sensors (Traffic Ear) and vehicle telematics data.” Sensors 23, no. 15 (2023): 6964.