Ghost Vehicle Detection

Complete Firmware Source Code

TM4C123GH6PM — Keil MDK v5 — Embedded C (Register-Level)

delay.h

Header for the millisecond delay utility.

#ifndef DELAY_H

#define DELAY_H

#include <stdint.h>

void Delay_ms(uint32_t ms);

#endif

delay.c

Simple busy-wait millisecond delay, used for OLED timing, startup pacing, and the main control loop.

#include "delay.h"

#include "TM4C123GH6PM.h"

void Delay_ms(uint32_t ms) {

volatile uint32_t count;

uint32_t i;

for (i = 0; i < ms; i++) {

count = 3180;

while (count--);

}

}

adc.h

Header for the 3-channel acoustic ADC driver (Front, Back-Right, Back-Left microphones).

#ifndef ADC_H

#define ADC_H

#include <stdint.h>

void ADC0_Init3Ch(void);

void ADC0_ReadAll(uint16_t *buf);   // buf[0]=Front, buf[1]=BackRight, buf[2]=BackLeft

#endif

adc.c

Configures ADC0 Sample Sequencer 1 to read all three MAX4466 microphone channels (PE3/PE2/PE1) in one triggered sequence. Channel order (SSMUX1 = 0x00000210) was corrected after an earlier version accidentally sampled an unconnected pin first.

#include "adc.h"

#include "TM4C123GH6PM.h"

void ADC0_Init3Ch(void) {

SYSCTL->RCGCADC  |= (1<<0);

SYSCTL->RCGCGPIO |= (1<<4);

while(!(SYSCTL->PRGPIO & (1<<4)));

GPIOE->AFSEL |= 0x0E;             // PE1,PE2,PE3 alternate function

GPIOE->DEN   &= ~0x0E;

GPIOE->AMSEL |= 0x0E;

ADC0->ACTSS  &= ~(1<<1);          // disable SS1 to configure

ADC0->EMUX   &= ~0x00F0;          // SS1 = software trigger

ADC0->SSMUX1  = 0x00000210;       // sample0=AIN0(Front,PE3), sample1=AIN1(BackRight,PE2), sample2=AIN2(BackLeft,PE1)

ADC0->SSCTL1  = (1<<10);          // END2 set on 3rd sample

ADC0->IM     &= ~(1<<1);          // polling, no interrupt

ADC0->ACTSS  |= (1<<1);

}

void ADC0_ReadAll(uint16_t *buf) {

ADC0->PSSI |= (1<<1);             // trigger SS1

while(!(ADC0->RIS & (1<<1)));     // wait for conversion sequence done

buf[0] = ADC0->SSFIFO1 & 0xFFF;   // Front

buf[1] = ADC0->SSFIFO1 & 0xFFF;   // Back-Right

buf[2] = ADC0->SSFIFO1 & 0xFFF;   // Back-Left

ADC0->ISC = (1<<1);               // clear completion flag

}

i2c.h

Header for the I2C0 master driver used to talk to the SSD1306 OLED.

#ifndef I2C_H

#define I2C_H

#include <stdint.h>

void I2C0_Init(void);

void I2C0_Send(uint8_t slaveAddr, uint8_t data);

#endif

i2c.c

Configures I2C0 (PB2=SCL, PB3=SDA) for approximately 100kHz master-mode operation.

#include "i2c.h"

#include "TM4C123GH6PM.h"

void I2C0_Init(void) {

SYSCTL->RCGCI2C  |= (1<<0);

SYSCTL->RCGCGPIO |= (1<<1);

while(!(SYSCTL->PRGPIO & (1<<1)));

GPIOB->AFSEL |= (1<<2)|(1<<3);    // PB2,PB3 alternate function

GPIOB->PCTL &= ~0x0000FF00;

GPIOB->PCTL |=  0x00003300;       // I2C0 SCL/SDA mux value

GPIOB->ODR  |= (1<<3);            // open-drain on SDA

GPIOB->DEN  |= (1<<2)|(1<<3);

I2C0->MCR = 0x10;                 // master mode

I2C0->MTPR = 7;                   // ~100kHz @ 16MHz sysclk

}

void I2C0_Send(uint8_t slaveAddr, uint8_t data) {

I2C0->MSA = (slaveAddr << 1);

I2C0->MDR = data;

I2C0->MCS = 0x07;                 // START+RUN+STOP

while(I2C0->MCS & 0x01);

}

led.h

Header for the external LED driver (PB4, via current-limiting resistor).

#ifndef LED_H

#define LED_H

void LED_Init(void);

void LED_On(void);

void LED_Off(void);

#endif

led.c

Simple GPIO driver for the external warning LED on PB4.

#include "led.h"

#include "TM4C123GH6PM.h"

void LED_Init(void) {

SYSCTL->RCGCGPIO |= (1<<1);       // Port B clock

while(!(SYSCTL->PRGPIO & (1<<1)));

GPIOB->DIR |= (1<<4);             // PB4 output

GPIOB->DEN |= (1<<4);

}

void LED_On(void)  { GPIOB->DATA |= (1<<4); }

void LED_Off(void) { GPIOB->DATA &= ~(1<<4); }

buzzer.h

Header for the active buzzer driver (PB5).

#ifndef BUZZER_H

#define BUZZER_H

void Buzzer_Init(void);

void Buzzer_On(void);

void Buzzer_Off(void);

#endif

buzzer.c

Simple GPIO driver for the active buzzer on PB5, reserved for critical-proximity alerts.

#include "buzzer.h"

#include "TM4C123GH6PM.h"

void Buzzer_Init(void) {

SYSCTL->RCGCGPIO |= (1<<1);       // Port B clock

while(!(SYSCTL->PRGPIO & (1<<1)));

GPIOB->DIR |= (1<<5);             // PB5 output

GPIOB->DEN |= (1<<5);

}

void Buzzer_On(void)  { GPIOB->DATA |= (1<<5); }

void Buzzer_Off(void) { GPIOB->DATA &= ~(1<<5); }

font.h

Header exposing the 5x7 pixel font table used by the OLED driver.

#ifndef FONT_H

#define FONT_H

#include <stdint.h>

extern const uint8_t font5x7[][5];

#endif

font.c

Complete 5x7 pixel font table covering printable ASCII 32 (space) through 95 (underscore) — digits, uppercase letters, and common symbols.

#include "font.h"

// 5x7 font table, ASCII 32 (space) to 95 (underscore)

const uint8_t font5x7[][5] = {

{0x00,0x00,0x00,0x00,0x00}, // 32 space

{0x00,0x00,0x5F,0x00,0x00}, // 33 !

{0x00,0x07,0x00,0x07,0x00}, // 34 "

{0x14,0x7F,0x14,0x7F,0x14}, // 35 #

{0x24,0x2A,0x7F,0x2A,0x12}, // 36 $

{0x23,0x13,0x08,0x64,0x62}, // 37 %

{0x36,0x49,0x56,0x20,0x50}, // 38 &

{0x00,0x08,0x07,0x03,0x00}, // 39 '

{0x00,0x1C,0x22,0x41,0x00}, // 40 (

{0x00,0x41,0x22,0x1C,0x00}, // 41 )

{0x2A,0x1C,0x7F,0x1C,0x2A}, // 42 *

{0x08,0x08,0x3E,0x08,0x08}, // 43 +

{0x00,0x80,0x70,0x30,0x00}, // 44 ,

{0x08,0x08,0x08,0x08,0x08}, // 45 -

{0x00,0x00,0x60,0x60,0x00}, // 46 .

{0x20,0x10,0x08,0x04,0x02}, // 47 /

{0x3E,0x51,0x49,0x45,0x3E}, // 48 0

{0x00,0x42,0x7F,0x40,0x00}, // 49 1

{0x42,0x61,0x51,0x49,0x46}, // 50 2

{0x21,0x41,0x45,0x4B,0x31}, // 51 3

{0x18,0x14,0x12,0x7F,0x10}, // 52 4

{0x27,0x45,0x45,0x45,0x39}, // 53 5

{0x3C,0x4A,0x49,0x49,0x30}, // 54 6

{0x01,0x71,0x09,0x05,0x03}, // 55 7

{0x36,0x49,0x49,0x49,0x36}, // 56 8

{0x06,0x49,0x49,0x29,0x1E}, // 57 9

{0x00,0x36,0x36,0x00,0x00}, // 58 :

{0x00,0x56,0x36,0x00,0x00}, // 59 ;

{0x08,0x14,0x22,0x41,0x00}, // 60 <

{0x14,0x14,0x14,0x14,0x14}, // 61 =

{0x00,0x41,0x22,0x14,0x08}, // 62 >

{0x02,0x01,0x51,0x09,0x06}, // 63 ?

{0x32,0x49,0x79,0x41,0x3E}, // 64 @

{0x7E,0x11,0x11,0x11,0x7E}, // 65 A

{0x7F,0x49,0x49,0x49,0x36}, // 66 B

{0x3E,0x41,0x41,0x41,0x22}, // 67 C

{0x7F,0x41,0x41,0x22,0x1C}, // 68 D

{0x7F,0x49,0x49,0x49,0x41}, // 69 E

{0x7F,0x09,0x09,0x09,0x01}, // 70 F

{0x3E,0x41,0x49,0x49,0x7A}, // 71 G

{0x7F,0x08,0x08,0x08,0x7F}, // 72 H

{0x00,0x41,0x7F,0x41,0x00}, // 73 I

{0x20,0x40,0x41,0x3F,0x01}, // 74 J

{0x7F,0x08,0x14,0x22,0x41}, // 75 K

{0x7F,0x40,0x40,0x40,0x40}, // 76 L

{0x7F,0x02,0x0C,0x02,0x7F}, // 77 M

{0x7F,0x04,0x08,0x10,0x7F}, // 78 N

{0x3E,0x41,0x41,0x41,0x3E}, // 79 O

{0x7F,0x09,0x09,0x09,0x06}, // 80 P

{0x3E,0x41,0x51,0x21,0x5E}, // 81 Q

{0x7F,0x09,0x19,0x29,0x46}, // 82 R

{0x46,0x49,0x49,0x49,0x31}, // 83 S

{0x01,0x01,0x7F,0x01,0x01}, // 84 T

{0x3F,0x40,0x40,0x40,0x3F}, // 85 U

{0x1F,0x20,0x40,0x20,0x1F}, // 86 V

{0x3F,0x40,0x38,0x40,0x3F}, // 87 W

{0x63,0x14,0x08,0x14,0x63}, // 88 X

{0x07,0x08,0x70,0x08,0x07}, // 89 Y

{0x61,0x51,0x49,0x45,0x43}, // 90 Z

{0x00,0x7F,0x41,0x41,0x00}, // 91 [

{0x02,0x04,0x08,0x10,0x20}, // 92 backslash

{0x00,0x41,0x41,0x7F,0x00}, // 93 ]

{0x04,0x02,0x01,0x02,0x04}, // 94 ^

{0x40,0x40,0x40,0x40,0x40}, // 95 _

};

oled.h

Header for the SSD1306 OLED driver.

#ifndef OLED_H

#define OLED_H

#include <stdint.h>

void OLED_Init(void);

void OLED_Clear(void);

void OLED_SetCursor(uint8_t page, uint8_t col);

void OLED_PrintChar(char c);

void OLED_PrintString(char *str);

void OLED_PrintNumber(uint16_t number);

#endif

oled.c

Register-level SSD1306 driver over I2C0 (address 0x3C): initialisation sequence, screen clear, cursor addressing, and text rendering via the 5x7 font table.

#include "TM4C123GH6PM.h"

#include "oled.h"

#include "i2c.h"

#include "font.h"

#define OLED_ADDR 0x3C

static void OLED_Command(uint8_t cmd) {

I2C0->MSA = (OLED_ADDR << 1);

I2C0->MDR = 0x00;                 // control byte: command

I2C0->MCS = 0x03;                 // START+RUN, no stop yet

while(I2C0->MCS & 0x01);

I2C0->MDR = cmd;

I2C0->MCS = 0x05;                 // RUN+STOP

while(I2C0->MCS & 0x01);

}

static void OLED_Data(uint8_t data) {

I2C0->MSA = (OLED_ADDR << 1);

I2C0->MDR = 0x40;                 // control byte: data

I2C0->MCS = 0x03;

while(I2C0->MCS & 0x01);

I2C0->MDR = data;

I2C0->MCS = 0x05;

while(I2C0->MCS & 0x01);

}

void OLED_Init(void) {

OLED_Command(0xAE); // display off

OLED_Command(0xD5); OLED_Command(0x80);

OLED_Command(0xA8); OLED_Command(0x3F);

OLED_Command(0xD3); OLED_Command(0x00);

OLED_Command(0x40);

OLED_Command(0x8D); OLED_Command(0x14); // charge pump on

OLED_Command(0x20); OLED_Command(0x00); // horizontal addressing

OLED_Command(0xA1); // segment remap

OLED_Command(0xC8); // COM scan direction

OLED_Command(0xDA); OLED_Command(0x12);

OLED_Command(0x81); OLED_Command(0xCF);

OLED_Command(0xD9); OLED_Command(0xF1);

OLED_Command(0xDB); OLED_Command(0x40);

OLED_Command(0xA4);

OLED_Command(0xA6);

OLED_Command(0xAF); // display ON

}

void OLED_Clear(void) {

uint8_t page, col;

for (page = 0; page < 8; page++) {

OLED_SetCursor(page, 0);

for (col = 0; col < 128; col++)

OLED_Data(0x00);

}

}

void OLED_SetCursor(uint8_t page, uint8_t col) {

OLED_Command(0xB0 + page);

OLED_Command(0x00 + (col & 0x0F));

OLED_Command(0x10 + ((col >> 4) & 0x0F));

}

void OLED_PrintChar(char c) {

uint8_t index = c - 32;

uint8_t i;

for (i = 0; i < 5; i++)

OLED_Data(font5x7[index][i]);

OLED_Data(0x00); // 1px spacing

}

void OLED_PrintString(char *str) {

while (*str) OLED_PrintChar(*str++);

}

void OLED_PrintNumber(uint16_t number) {

char buf[6];

int i = 0;

if (number == 0) { OLED_PrintChar('0'); return; }

while (number > 0) { buf[i++] = (number % 10) + '0'; number /= 10; }

while (i > 0) OLED_PrintChar(buf[--i]);

}

ultrasonic.h

Header for the HC-SR04 ultrasonic ranging driver.

#ifndef ULTRASONIC_H

#define ULTRASONIC_H

#include <stdint.h>

void Ultrasonic_Init(void);

uint16_t Ultrasonic_GetDistanceCM(void);   // returns 0xFFFF if no echo received

#endif

ultrasonic.c

HC-SR04 time-of-flight ranging driver using Timer0 as a free-running microsecond counter (TRIG=PB6, ECHO=PB7). Includes a proper Delay_us() microsecond delay (replacing an earlier unreliable busy-wait loop) and a timeout that returns 0xFFFF ("NO SIGNAL") if the echo pin is disconnected or floating.

#include "ultrasonic.h"

#include "TM4C123GH6PM.h"

#define TRIG_PIN (1<<6)   // PB6

#define ECHO_PIN (1<<7)   // PB7 (via voltage divider or 3.3V-powered sensor)

void Ultrasonic_Init(void) {

SYSCTL->RCGCGPIO |= (1<<1);         // Port B clock

while(!(SYSCTL->PRGPIO & (1<<1)));

GPIOB->DIR |= TRIG_PIN;             // TRIG = output

GPIOB->DIR &= ~ECHO_PIN;            // ECHO = input

GPIOB->DEN |= TRIG_PIN | ECHO_PIN;

// Timer0 configured as a free-running microsecond-resolution counter

SYSCTL->RCGCTIMER |= (1<<0);

while(!(SYSCTL->PRTIMER & (1<<0)));

TIMER0->CTL  &= ~(1<<0);            // disable Timer0A to configure

TIMER0->CFG   = 0x00;               // 32-bit timer

TIMER0->TAMR  = 0x02;               // periodic up-count mode

TIMER0->TAILR = 0xFFFFFFFF;         // max count

TIMER0->CTL  |= (1<<0);             // enable Timer0A

}

static uint32_t GetTicks(void) {

return TIMER0->TAV;

}

// Assumes default 16MHz system clock (no PLL configured elsewhere in project)

static void Delay_us(uint32_t us) {

uint32_t start = TIMER0->TAV;

uint32_t ticks = us * 16;   // 16 ticks per microsecond @ 16MHz

while ((TIMER0->TAV - start) < ticks);

}

uint16_t Ultrasonic_GetDistanceCM(void) {

uint32_t startTicks, endTicks, pulseTicks, pulse_us;

uint32_t timeout;

// Send a clean 10us trigger pulse

GPIOB->DATA &= ~TRIG_PIN;

Delay_us(2);

GPIOB->DATA |= TRIG_PIN;

Delay_us(10);

GPIOB->DATA &= ~TRIG_PIN;

// Wait for ECHO to go HIGH (timeout guards against a stuck/disconnected sensor)

timeout = 1000000;

while(!(GPIOB->DATA & ECHO_PIN)) {

if(--timeout == 0) return 0xFFFF;

}

startTicks = GetTicks();

// Wait for ECHO to go LOW

timeout = 1000000;

while(GPIOB->DATA & ECHO_PIN) {

if(--timeout == 0) return 0xFFFF;

}

endTicks = GetTicks();

pulseTicks = endTicks - startTicks;

pulse_us = pulseTicks / 16;              // 16 ticks = 1 microsecond @ 16MHz

return (uint16_t)(pulse_us / 58);        // standard HC-SR04: distance(cm) = time(us)/58

}

main.c

Top-level control loop: initialises every peripheral (buzzer forced off first), then continuously reads the 3 acoustic channels and the ultrasonic distance, updates the OLED with DIST/SND/status, and drives the LED and buzzer via the distance-primary sensor-fusion logic. Thresholds are calibrated for the 35cm RC-car test box.

#include "TM4C123GH6PM.h"

#include "adc.h"

#include "i2c.h"

#include "oled.h"

#include "buzzer.h"

#include "led.h"

#include "delay.h"

#include "ultrasonic.h"

typedef enum { DIR_FRONT, DIR_BACKRIGHT, DIR_BACKLEFT } Direction_t;

Direction_t GetDirection(uint16_t *adc, uint16_t *maxVal) {

uint8_t maxIdx = 0;

uint8_t i;

for (i = 1; i < 3; i++)

if (adc[i] > adc[maxIdx]) maxIdx = i;

*maxVal = adc[maxIdx];

return (Direction_t)maxIdx;

}

/* Distance thresholds for a 35cm box.

Ultrasonic distance is the primary trigger.

Buzzer only activates when distCM < 10 (critical proximity). */

void UpdateAlerts(uint16_t distCM) {

if (distCM > 30)       { Buzzer_Off(); LED_Off(); }

else if (distCM > 10)  { Buzzer_Off(); LED_On();  }

else                   { Buzzer_On();  LED_On();  }   // <10cm = critical

}

const char* GetStatusText(uint16_t distCM) {

if (distCM == 0xFFFF)   return "NO SIGNAL";

if (distCM > 30)        return "MONITORING...";

else if (distCM > 20)   return "CAR DETECTED";

else if (distCM > 10)   return "APPROACHING!";

else                    return "!!CRITICAL!!";

}

int main(void) {

uint16_t adc[3];

uint16_t maxVal;

uint16_t distCM;

// Buzzer initialized and forced silent FIRST, before anything else runs

Buzzer_Init();

Buzzer_Off();

ADC0_Init3Ch();

I2C0_Init();

OLED_Init();

LED_Init();

Ultrasonic_Init();

OLED_Clear();

OLED_SetCursor(0,0);

OLED_PrintString("GHOST VEHICLE");

OLED_SetCursor(1,0);

OLED_PrintString("FUSION MODE");

Delay_ms(1500);

while (1) {

ADC0_ReadAll(adc);

GetDirection(adc, &maxVal);   // acoustic level shown as SND: confirmation only

distCM = Ultrasonic_GetDistanceCM();

OLED_Clear();

OLED_SetCursor(0,0);

OLED_PrintString("DIST:");

if (distCM == 0xFFFF) OLED_PrintString("NOECHO");

else { OLED_PrintNumber(distCM); OLED_PrintString("cm"); }

OLED_SetCursor(1,0);

OLED_PrintString("SND:");

OLED_PrintNumber(maxVal);

OLED_SetCursor(2,0);

OLED_PrintString((char*)GetStatusText(distCM));

UpdateAlerts(distCM);

Delay_ms(150);

}

}